[NOTE:    This version is from optical character recognition of a scanned copy.    It has been proofread but may still contain typographical errors.    Oct. 20, 2002]

LEGAL OPEN BURNING AND ON-PREMISES BURIAL OF SOLID WASTE

IN ST. LAWRENCE COUNTY, NY:

THE ENVIRONMENTAL IMPACTS

St. Lawrence County Planning Office

Canton, NY

February 5, 1993

LEGAL OPEN BURNING AND ON-PREMISES BURIAL OF SOLID WASTE

CAPSULE SUMMARY

This study of the impacts of legal open burning and burial of solid waste was undertaken by direction of the St. Lawrence County Board of Legislators. The County Planning Office sought to determine whether the environmental impacts are severe enough to warrant County or local regulation.    This is one element in a broader study of the environmental, health and economic impacts of these practices.

Existing State law and regulations allow burning and burial, within limits.    Where nuisance conditions exist as a result of these practices, current law enables local or state authorities to act pursuant to Public Health Law or Environmental Conservation Law. The report cites these authorities in detail.

Only secondary research techniques were used to determine the environmental impact, specifically, a search of the technical and scientific literature and contacts with knowledgeable authorities. The study concludes there is insufficient primary research on the impacts of "barrel burning" and on-premises burial to form a basis for regulation based only on proven environmental impact. This is not usually the only basis for regulating, however.

The study reports the extent of on-premises, open burning. It did not prove feasible to assess the extent of at-home burial of solid waste. Using an established geographical sampling technique, the Planning Office concludes that a minimum 48.2%, or 9926 rural households make some use of "burn barrels" or similar means of incineration at home.    This quantitative assessment will be important to an analysis of the health effects and the economic and fiscal impacts of at-home waste disposal.

LEGAL OPEN BURNING AND ON-PREMISES BURIAL OF SOLID WASTE

IN ST. LAWRENCE COUNTY, NY:

THE ENVIRONMENTAL IMPACTS

St. Lawrence County Planning Office

February 5, 1993

1.	INTRODUCTION: BASIS FOR THE STUDY

The St. Lawrence County Board of Legislators passed resolution No.234-92 on Oct.12. 1992), Initiating a Study of the Environmental, Economic and Health Impacts of Open Burning and On-Premises Burial of Solid Waste, which called upon county departments to work under the coordination of the Director of the Solid Waste Department to conduct a study and make recommendations concerning the regulation or prohibition of these historic practices.    The departments involved were understood to be Planning, Public Health and Solid Waste. The Planning Office took its tasks to be 1) determining the nature of environmental problems associated with open burning and burial, especially air pollution and groundwater pollution, and 2) estimating the extent of the two practices to provide a basis for a waste quantity estimate for use in the economic and health impact assessments. Information produced by the Planning Office was therefore considered to be the initial phase of the overall study.    Information on the environmental impacts and the extent of the practices directly contributes to the Public Health Department's study of health impacts. The Planning Office's estimate of the extent of the practices is essential for the Solid Waste Department to evaluate the economic impact of diversion from the main waste stream on haulers' income and the tipping fee, among other possible impacts.

This report is based upon searches of the technical literature, personal contacts and other inquiries to agencies, and original field research by the County Planning Office. Sources and research techniques are indicated.

2.	DEFINITIONS

For the purposes of this study, the following definitions are used:

1.	On-Premises Burial: The placement of solid waste generated on the premises of either single family residential lots or farms, either on or below the surface of the earth, for the purpose of final disposal.    Industrial or commercial landfills requiring a permit under 6NYCRR Part 360 are not included in this definition.

2.	Open Burning: Combustion or solid waste in an uncontrolled manner without the benefit of air pollution control devices which are capable of substantially reducing emissions of particulates, gasses or flyash to levels which are in compliance with applicable New York State air quality regulations.    Typically this practice is carried out in 55-gallon drum "burn barrels" or other similar primitive devices which often fail to provide adequate combustion air.

3.	Solid Waste: This term has the same meaning as the definition in 6 NYCRR Part 360, which includes putrescible and non-putrescible materials or substances that are discharged, including but not limited to garbage, refuse, industrial and commercial waste, sludges, rubbish, tires, ashes, incinerator residue, construction and demolition debris, discarded automobiles and offal.

3.	THE REGULATORY SITUATION: CURRENT LAWS AND REGULATIONS

The open burning of solid waste (open burning) is addressed in New York State law and regulations, as is on-premises burial of solid waste. State law regulating open burning and burial is found in the Environmental Conservation Law (ECL) and the Public Health Law (PHL). The full texts are available from the Law Library. There are few local laws and ordinances regulating these practices in St. Lawrence County. The Town and Village of Edwards and the Villages of Morristown and Canton (possibly others) have language in their land-use regulations which limit on-site disposal of solid waste.

A.	THE REGULATORY SITUATION: OPEN BURNING

Prohibitions on Open Burning

Open burning is generally regulated by the State. Regulations are found in 6NYCRR Part 215 "Open Fires". The statutory authority is PHL §1271 and §1276 and ECL §15 and §17.

Statute controls the burning of "rubbish", defined as: "Solid or liquid waste material, including but not limited tot paper and paper products: rags; trees, or leaves, needles and branches therefrom; vines; lawn and garden debris; furniture; cans; crockery; plastics; cartons; chemicals; paint; grease; sludges; oils and other petroleum products; wood; sawdust, demolition materials; tires; and automobiles and other vehicles and parts, for junk, salvage, or disposal. Rubbish shall not include garbage, incinerator residue, street sweepings, dead animals, or offal."

Except as permitted by Part 215.3, "...no person shall burn, cause, suffer, allow, or permit the burning in an open fire:

a)	of garbage (the animal and vegetable waste resulting from the handling, preparation, cooking and serving of food);

b)	of refuse at a refuse disposal area (landfills, sanitary landfills and dumps);

c)	of rubbish for salvage;

d)	for on-site disposal, of rubbish generated by residential activities in any city or village; or in any town with a total town population, including incorporated or unincorporated areas, of greater than 20,000;

e)	for on-site disposal, of rubbish generated by industrial or commercial activities other than agricultural;

f)	of rubbish generated by land clearing or demolition for the erection of any structure; for the construction or modification of any highway, railroad, power or communication line, or pipeline; or for the development or modification of a recreational area or park;

g)	of refuse during an air pollution episode, in an area for which such air pollution has been designated."

This section of 215.3 allows the Commissioner of the DEC to grant permits for special situations in which open burning has been shown to be necessary, provided air quality standards will not be needlessly jeopardized, such as burning tree limbs following an ice storm, for example.

Forest Fire Control

Each town in St. Lawrence County is either a designated "fire town" or a town in a "fire district". Under the statutory authority (ECL Art. 9, Title II, Sections 9-1105,9-1107, 9-1109) the following regulations apply:

Section 9-1105, 2. "No person shall deposit, and leave in any of the fire towns, or in any town included in a fire district as defined in subdivision 2 of Section 9-1109, brush or inflammable material [emphasis added] upon the right-of-way of highways."

Section 9-1105,4. "In any of the fire towns, or in any town included in a fire district as defined in subdivision 2 of Section 9-1109, brush, logs, slash or other inflammable material [emphasis added] shall not be left or allowed to remain on land within 25 feet of the right-of-way of a railroad or within 20 feet of the right-of-way of a public highway."

Section 9-1105, 5. " No person shall set or cause to be set fire for purpose of burning logs, sawdust, brush, stumps, dry grass or other debris [debris not defined], in any of the fire towns [Clare, Clifton, Colton, Fine, Hopkinton, Parishville, Piercefield, Pitcairn], without first having obtained from the department [DEC] a written permit so to do."

Air Quality

Air quality is regulated under 6 NYCRR Part 211.2 "Air Pollution Prohibited" pursuant to statutory authority in ECL Sections 3-0301, 19-0301, 19-0303, 19-0305.

Part 211.2 states, "No person shall cause or allow emissions of air contaminants to the outdoor atmosphere of such quantity, characteristic or duration which are injurious to human, plant or animal life or to property, or which unreasonably interfere with the comfortable enjoyment of life or property. Notwithstanding the existence of specific air quality standards or emission limits, this prohibition applies, but is not limited to, any particulate, fume, gas, mist, odor, smoke, vapor, pollen, toxic or deleterious emission, either alone or in combination with others."

Local laws are also possible, pursuant to statutory authority found in ECL Art. 19. Title 7, Section 19-0709, which allows local governments to enact their own laws to address problems of air pollution, provided they are neither contrary to nor no less stringent than ECL Art. 19 or 6 NYCRR Part 200.

B.	THE REGULATORY SITUATION: ON PREMISES BURIAL

Regulations for on-premises disposal of solid waste are found in 6 NYCRR 360-1.7(b), which is based on ECL Sections 1-0101, 3-0301, 8-0113, 19-0301, 19-0306, 23-2305, 23-2307, 27-0101, 27-0106, 27-0305, 27-0703, 27-0704, 27-0705, 27-0911, 27-1317, 27-1515, 52-0107, 52-0505 and 70-0107.

Some burial is exempt from regulation. Among the solid waste management facilities which are exempt from the need to obtain a Part 360 permit (i.e., are "allowed") are "disposal areas located within the property boundaries of a single family residence or farm for solid waste generated from that residence or farm" and:

1.	Disposal areas for waste pesticides by the farmer who used them if the farmer complies with sections 325.4 and 325.5 of this Title.

2.	Disposal areas located within the property boundaries of a farm for crop residues, animal manure, and animal carcasses and parts generated from a farm.

3.	Transfer, storage, treatment, incinerator, and processing facilities, except composting facilities, located at a single or multiple family residence, school, park, industry, hospital, commercial establishment, or farm and used exclusively for the management of solid waste generated at that location.

4.	Transfer, storage, treatment, incinerator, and processing facilities, except composting facilities, located at publicly-owned treatment works, and usedexclusively for sewage sludge provided solid waste is stored less than 18 months.

5.	The initial site used for the collection of household hazardous waste for a clean-up day or similar event sponsored by a community or governmental organization on a not-for-profit basis, and designed to assist the public in disposing of unwanted pesticides and other household hazardous waste if: [several conditions are set].

6.	A transporter storing shipments of non-putrescible industrial and commercial waste, other than infectious waste, in containers or vehicles at its own transfer facility for a period of five calendar days or less, if the transporter complies with the following requirements: [several requirements are listed].

7.	A facility that exclusively treats wastewater that is regulated under Parts 750 and 757 of this Title."

C.	CONTROL OF NUISANCE

Part 8 of Chapter 1 of the State Sanitary Code deals with "Nuisances Which May Affect Life and Health".    The local Health Officer is empowered to investigate a nuisance following a complaint and take steps to secure its voluntary abatement. If the nuisance is not abated a report is prepared and filed with the local board of health which then makes a determination of nuisance, issues a show-cause notice to the perpetrator, holds a hearing and can then direct abatement of the nuisance and impose penalties. Local governments can vary' the procedure from that described in the State Sanitary Code after the step at which the local Health Officer reports.

D.	SUMMARY AND CONCLUSIONS ON THE REGULATORY SITUATION

Open Burning. The report cites several references in state law and regulations that relate to control of open burning not only of solid waste (rubbish), but also other waste materials such as slash and brush, etc. Broad authority already exists to control any nuisance or problematical open burning, including "barrel burning". Nevertheless, the County or individual towns may enact their own laws and provide their own enforcement if they consider that the State does not or cannot provide sufficient protection under existing law.    A first step might be obtaining a formal statement from the State (DEC) regarding that agency's ability to enforce the current state laws and regulations. Based on the response, the County or towns would then decide whether open burning presents enough of a problem to warrant additional regulation.

On-Premises Burial. There appears to be no precedent or specific authority in New York State Law to restrict this activity. Should a particular instance be shown to produce a demonstrable community health threat, the means exists to deal with it effectively.

4.    ENVIRONMENTAL IMPACT

It is the Planning Office's understanding that the Public Health Department will be investigating the human health implications of on-premises burial and open burning.    The Planning Office has little expertise in the area of public health.    While this report on environmental impact contains information similar to what may be found in a health assessment, the health impact information should be regarded as incidental to a search of relevant scientific literature.

On-Premises Burial

No research was located which indicates whether or not on-premises burial of solid wastes poses a threat to the environment on a widespread, cumulative basis. In specific situations the burial of significant quantities of wastes in immediate proximity to water bodies or improperly sealed water wells can cause pollution which may result in impacts to wildlife or humans. In the Planning Office's experience such situations are mainly encountered where untidy commercial establishments adjoin residential lots. For the most part these situations have been problems of aesthetic nuisance, not demonstrated health threats. Communities concerned with this sort of problem have the ability to anticipate them through land-use regulations, such as zoning or site plan review, or through special-purpose local laws or ordinances. Individual situations can be addressed by the municipal health officer acting on a complaint.

Open Burning

The environmental health impact of open burning is also not well documented in the literature. One article dating from 1967 reported on a study which compared emissions from the open burning of "municipal refuse", "landscape refuse" and "automobile components" for carbon dioxide, carbon monoxide, hydrogen cyanide, formaldehyde, organic acids, nitrogen oxides, particulates and polynuclear hydrocarbons (PNH) (3). There were two significant findings of this study. The emission values for most pollutants were higher compared to emissions from small, specially-designed incinerators. Burning "auto components" produced many more particulates (six times more) and a factor of 100 times more PNH than either municipal or landscape refuse. Overall, "atmospheric emissions from open burning are characterized by high emissions of products of incomplete combustion."    One limitation of the study was that it did not simulate the oxygen-starved conditions commonly found in backyard burn barrels and therefore probably systematically underestimated real-world emission rates.

While the Planning Office study concentrates on burning household waste, findings on some other common burning practices are instructive, if only for comparison. Leaf and debris burning resulting from yard clearing activities without the municipal solid waste component has received some scientific research attention (1,2,3,4,5). Generally, the combustion of biomass in less-than-optimal conditions produces quantities of particulates, carbon monoxide hydrocarbons and PNH. Some of these PNLI compounds, also called polyaromatic hydrocarbons (PAN), have mutagenic (causing mutations in genetic material) potential.    Under the right conditions of exposure, the open burning of leaves produces contaminants in the atmosphere in sufficient quantity, characteristics and duration to be harmful to human health (4). A study in Oregon found that burning yard debris in the Spring can contribute as much particulate to the ambient air as woodburning does during the winter (2).

It is a matter of common experience that burning leaves, brush or trash in confined atmospheric conditions, such as during inversions or near clusters of buildings which impede airflow, is a nuisance. Unfortunately, this common experience does not lend itself well to systematic, scientific study. In short, problems are situational and the variables of waste type, quantity, combustion frequency and duration are many. What proves to be a noxious situation in a trailer park or hamlet or in a "frost pocket" during the winter may not be a problem where the landscape more open. A letter from James M. Melius, M.D., Dr. of Public Health, who is the Director of the Division of Occupational Health and Environmental Epidemiology for the New York State Department of Health to Martha Clarvoe, President of the League of Women Voters, Cooperstown, New York conveys his experience and knowledge with the specific problem of "burn barrels"(6). Excerpts follow:

"While almost any combustion process poses potential environmental health risks, I believe that the increasing use of these "burn barrels" poses a potentially more serious local problem. Much of our household and commercial waste consists of plastic and other synthetic materials. These materials when burned may release a large number of toxic chemicals not ordinarily found at high concentrations in the burning of wood and other natural materials. These toxic chemicals may include toluene diisocyanate (a potent respiratory irritant which may cause asthma and other pulmonary problems); benzene (a chemical known to cause leukemia in exposed workers); nitrogen oxides (which may cause pulmonary damage); nitrile compounds (which are metabolic poisons and carcinogens); and many others.

These chemicals as well as other combustion products (polynuclear aromatic compounds, etc.) may significantly impact the health of nearby residents.    Elderly people with respiratory or bean disease, asthmatics, and young children are more susceptible to effects from this type of exposure.    For example, smoke from this type of burning can significantly aggravate asthma or emphysema leading to serious acute medical problems. Local atmospheric conditions such as an atmospheric inversion. etc. may further exacerbate this situation."

Attempts by the staff to contact New York State agencies for information on the subject of open burning inevitably lead to the DEC's Division of Air, which claimed to have no information on the subject and referred the staff to the regional DEC office.    Previous conversations with regional DEC staff members have indicated that. while they are well aware of the problem, they know of no studies dealing with solid waste combustion in "backyard" situations.    Their experience with the problem is based on anecdotal accounts of specific problems. Staff phone conversations with USEPA staff have yielded no information. EPA has likewise apparently done no research on the subject since the burning of landfills was made illegal many years ago. The Waste Management Institute at Cornell University, which provided most of the references used in this report, has conducted no research of its own on this subject. In summary, the staff has made a substantial effort to contact agencies regarding the availability of research which documents environmental risk or damage. There is such a paucity of research that the Planning Office cannot conclude there is a technical or scientific basis for regulating "burn barrels".    It is the considered opinion of this office, however, that even rudimentary research would show that "barrel burning" contributes significantly to the deterioration of air quality.

5.	QUANTITIES OF WASTE BURNED OR BURIED

Randomly-selected town road segments throughout the County were surveyed for the purpose of obtaining an estimate of the number and percent of households using "burn barrels" and burying household waste on-premises. The result of the technique selected is termed a "stratified random sample". At least one road was taken randomly from each town's road list. A minimum of one road segment (the length of one named road) was driven in each town except Piercefield.    Two roads selected in Hermon proved to be seasonally maintained, with no residences. All sampled roads were outside incorporated Villages or the City of Ogdensburg. Staff members recorded the number of residences on each road segment and noted whether a burn barrel or burial site was evident. It became apparent quickly that it would not be possible to observe burial pits from the road. Given that sites in wooded or hilly terrain may contain barrels or pits which are not visible from the road, the resulting total and percentage is considered a conservative estimate, since all observation was from the road right-of-way. No attempts were made to enter private property.    Observations were made between mid November and late December, 1992. Lack of leaves on trees and shrubs probably enhanced visibility somewhat when compared to summer conditions.

This statistically rigorous and rather extensive sampling effort was done to ensure a representative look at the County as a whole and to ensure statistical reliability in extrapolating the sample results to the entire County. Anecdotally, the surveyors observed all manner of residential situations, leading them to have confidence that the results are representative.

A total of 427 residences were observed with 206, or 48.2%, showing evidence of open burning. Due to difficulties with visibility at many residences and the fact that some illegal open burning does occur in villages and the city of Ogdensburg, the actual frequency of open burning is undoubtedly over 50% for the County as a whole. The observed rate was over 60% in certain locations.

Using the 1990 total households count from the U.S. Census of 37,877 and subtracting the sum of the number of households from the Villages and the City of Ogdensburg (17,284)yielded a net non-incorporated total of 20,593 households.    Multiplying by the burning frequency (0.482) yields a minimum estimate of 9926 households which use open burning.

The overwhelming majority use "burn barrels", typically 55-gallon drums. A number use cement block fire pits. Burn barrels are typically located to the rear of the house, within convenient walking distance and along the property line. Occasionally, barrels could be found in the front yard. Where possible, barrels typically were seen to be located some distance from the living quarters.

REFERENCES

1.	Baubel, R.W.; Darley, E.F.; Schuck, E.A. (1969) Emissions from burning grass stubble and straw. JOURNAL OF THE AIR POLLUTION CONTROL ASSOCIATION 19 (7):

497-500.

2.	Edgerton, S.A.; KhaIil, M.A.K; Rasmussen, R.A. (1984) Estimates of air pollution from backyard burning. J. AIR POLLUTION CONTROL ASSOCIATION 34 (6): 661-664.

3.	Gerstle, R.W.; Kemnitz, D.A.    (1967) Atmospheric emissions from open burning. JOURNAL OF THE AIR POLLUTION CONTROL ASSOCIATION 17 (5): 324-327.

4.	Illinois Institute of Natural Resources. (1978) Advisory report on the potential health effects of leaf burning. IINR DOCUMENT 78/19. University of Illinois, Chicago, IL 60680.

5.	Kohn, R.E. (1977) A benefit-cost analysis to determine a population cutoff for a statewide ban on leaf burning. AIR POLLUTION CONTROL ASSOCIATION JOURNAL 27 (9): 887-889.

6.	Melius, J.M. (1991) Letter to Ms. Martha Clarvoe, President. League of Women Voters. July 9, 1991. NYSDOH Center for Environmental Health, 2 University Place, Albany, NY 12203-3399. 1 page.

STAFFING

Jon Montan designed the field survey, contacted knowledgeable individuals and wrote the draft report. Paul Stevenson searched the technical and scientific literature and assisted with field research. Richard Mooers provided overall direction, assisted with field research and wrote portions of the report. Robert Robinson and Keith Zimmerman assisted with field research.

February 5, 1993

3/EMC/BURN-BUR.RPT

ROAD SEGMENTS FOR OPEN BURNING SURVEY

Date:	11/19/92

Brasher:	Munson

Canton:	Ames, Sykes

Clare:	County Highway 27

Clifton:	Star Lake

Colton:	Morgan

DeKalb:	Pooler

DePeyster:	East

Edwards:	Sullvan

Fine:	Irish Hill

Fowler:	Battle Hill

Gouverneur:	Battle Hill

Hammond:	Wooster

Hermon:	Alverson

Hopkinton:	Beebe

Lawrence:	Barnage

Lisbon:	Balantine, Town Line

Louisville:	Wallace

Macomb:	Bishop

Madrid:	Thompson Corners- Chipman

Massena:	Carey

Morristown:	Atwood, Yankee

Norfolk:	Marsh

Oswegatchie:	Middle

Parishville:	Perkins

Piercefield:	Bancroft

Pierrepont:	Noyes

Pitcairn:	Osborne

Potsdam:	Hatch

Rossie:	Butler Rd.

Russell:	Backus

Stockholm:	Babylon, Skinnerville

Waddington:	Town Line

2/EMC/BURNSURV . TNS